- 49 BWRs with a total net electrical capacity of 49.6 GWe in worldwide operation in 2023

- 12 BWRs with a capacity of 12.3 GWe suspended the operation

- 2 BWRs with a capacity of 2.65 GWe are under construction

- 52 BWRs are permanently shut down with a capacity of 49.6 GWe

BWRs are an endangered system (they disappear)—most BWRs shut down in Japan after Fukushima.

4 types of advanced BWRs recently under design:

- ABWR-II –Advanced Boiling Water Reactor II (GE-Hitachi)

- ESBWR –Economic Simplified Boiling Water Reactor (GE-Hitachi)

- KERNA (AREVA)

- RMWR –Reduced-Moderation Water Reactor (JAEA) –with potential for breeding thanks to hard neutron spectrum

BWRs are simplified to reduce capital costs, but there is a trade-off as phenomena become more complex.

Reduced moderation water reactor:

- It can be used for breeding as it has a harder neutron spectrum.

- There has yet to be a significant interest in breeding.

ABWR was Introduced in 1991

- Experience in BWR operation

- Modular construction techniques

- Safety improvements (reduced core damage frequency)

- Design life 60 years

- No external Recirculation Loops (Reactor Internal Pumps)

Several types of reactor vessels exist:

- The pressure vessel (in PWR, BWR, and GCR)

- In CANDU calandria is a reactor vessel

- In RBMK –a steel shell containing the reactor core and graphite

Reactor vessels differ in shape and orientation:

- cylindrical or spherical (Magnox)

- vertical or horizontal (CANDU or PWR)

ABWR -Reactor Pressure Vessel

- Diameter 7.1 m,

- Height 21m

- Pressure 7 MPa

- Saturation temperature 286oC (Good margin for clad temperature 550oC)

- Steam separation at the top (Not possible to get CR from the top)

- ABWR RPV forged ring (forge to work heated metal by hammering or pressing it into shape)

- Manufacturing by forging rings:

  - Plates to rings

  - Rings to shell

- ABWR shell, bottom head:

Bottom head manufactured by forging from one piece - penetrations machined for motors, CRGTs IGTs

Functions of Reactor Pressure Vessel (RPV):

- houses reactor core (fuel)

- serves as a fission product barrier

- provides volume for water to keep the reactor core cooled

- Thermal sleeves: their function is to eliminate the risk of thermal fatigue at the inlet of (relatively) cold feedwater to RPV

  - Saturation temperature 286oC

  - Feedwater 215oC

  - 71oC difference,

  - thermal fatigue, cracks that can destroy steel elements in a year.

  - Flow into a Sparger then to downcomer

- Safety and performance elements all in one vessel

- Internal parts are more complex than PWR

  - PWR is one through circulation

  - BWR has an internal circulation

- Feedwater flows into a Sparger, then into the downcomer

- Core spray support and piping (Used for core cooling)

- Core shroud

  - Is not separating the flow of hot and cold coolant

  - The fuel canisters separate flow

- Control rods (CR)

  - Cross inside canister

  - Boron is not used because water boils

The reactor core is the nuclear reactor’s central part, having fuel, moderator (in thermal reactors), coolant, and reactivity/power control elements.

The core is characterized by the following:

- assembly geometry (square, hexagonal, cylindrical)

- fuel form (rods, pellets, plates)

- fuel material (U metal, UO2, UO2/PuO2, UO2/ThO2, UO2/MOX)

- refueling type (on-load, off-load)

- moderator material (light water, heavy water, carbon, none)

- fuel-clad material (magnesium alloys, stainless steel, zirconium alloys, advanced and composite materials, so-called accident tolerant fuel ATF)

- average fuel enrichment

- refueling frequency (12, 18, 24 months)

- active core diameter and height

- number of fissile fuel assemblies/bundles

- number of fertile fuel assemblies

- fuel and moderator weight

- average core power density (ratio of the thermal power and the volume of the core; W/m3)

- average fuel power density (ratio of the core thermal power and the volume of the fuel; W/m3)

- average fuel-specific power (ratio of the core thermal power and the fuel mass; W/kgU)

- fuel linear heat generation rate (thermal power per linear unit of fuel rod; W/m)

Main components of a BWR fuel assembly

  - 1. Top tie plate,

  - 2. Fuel rods,

  - 3. Channel,

  - 4. Water cross,

  - 5. Spacer grid,

  - 6. Bottom tie plate